CSE585/EE555:  Digital Image Processing II

Computer Project # 4:

Nonlinear Filtering and Anisotropic Diffusion

Yanxi Yang, Jiuchao Yin, Hongjie Liu

Date: 04/06/2020

- Objectives

- To classify and segment bipartite texture regions in an image.

- To understand the principles of Gabor filter and how to design a single Gabor filter on textured image to accomplish optimal segmentation.

- To learn the algorithm of Gaussian part (low pass filter) and complex sinusoid part of Gabor elementary function (GEF).

- To investigate the parameters specify the Gabor filter and smoothing filter.

- To get familiar with image 3-D plot from texture analysis from Gabor filter and smoothing filter.

- To extract boundaries between major textures regions and to explore the criteria of defining a good segmentation of texture.

- Methods

# Flow chart

# Figure1. flow chart of Gabor filter texture segmentation

Code structure:

main.m: the main code runs the 4 different tasks based on 4 images with various parameters settings, and output the grayscale images and 3D plot with m(x,y), m’(x,y) and final segmented results.

support functions:

- g.m: the Gaussian part of GEF This function is used to compute the circularly-symmetric Gaussian of one pixel. The inputs are the value of sigma and the x/y coordinate of the pixel. The input includes only x or only y since we compute convolution in x and y separately in main function. The output is the circularly-symmetric Gaussian value of that pixel. This function does not call any other functions.

- hx.m:  GEF of h(x) Since we compute GEF h(x, y) in x and y separately in main function, h(x, y) is separated into h1x and h2y. This function is used to compute the GEF in x, i.e. h1x. The inputs are F, theta, sigma, and x coordinate of the pixel. The output is the computation result of x coordinate. This function does not call any other functions.

- hy.m: GEF of h(y) This function is the same as hx.m. The only difference between them is that the inputs of this function are F, theta, sigma, and y coordinate of the pixel, not x. This function is used to compute the GEF in y, i.e. h2y and does not call any other functions.

- segment.m: This function is used to do segmentation with discriminative threshold of each texture for classification. The inputs are results after Gabor filter (m(x, y) or m'(x, y)), the original image, sigma, and threshold. The output is the visualized segmentation result. This function does not call any other functions.

- Algorithms

# To implement an optimal Gabor filter for texture segmentation, by looking at the output of m(x,y) that if it is a noisy step function for better discrimination and classification. Then we shall determine whether the GEF’s parameters are properly set or not.

# Algorithms are as following:

# By applying the Gabor filter:

# m (x,y) = [I(x,y) ** h(x,y)]

# where I denotes the input image, h is a GEF :

# h(x,y) = g(x, y) exp [j2F(xcos + ysin)] = g(x, y) exp [j2 (Ux + Vy)]

# where,  is the orientation of sinusoid.

# and g is a circularly-symmetric Gaussian:

# g(x,y) = 122  exp {-(x2+y2)22}

# The assumption of this project is that =0, so we can implement the GEF separably for x and y:

# h(x,y) = h1(x) h2(y)

# Thus, the GEF can be processed through three steps:

- i1(x,y) = i(x,y) * h1(x)

# = x'=-2σ+2σix-x', y h1(x')

- i2 (x,y) = i1(x,y) * h2(y)

# = y'=-2σ+2σi1x, y-y' h2(y')

- m(x,y) = | i2 (x,y)|

# Thus,  Gabor filter has  width of (4+1).

# To get easier for the segmentation, apply the smoothing filter:

# m’(x,y) = m(x,y) * g’(x,y)

# where g′ is another circular-symmetric Gaussian using a different  .

- Results

Since that values near the outer perimeter of the image cannot be completely processed, thus, when displaying results, we have zeroed out unprocessed perimeter areas.

Image1:

Figure 2. binary image of ‘texture1’

Figure 3. grayscale image of Gabor filter (left)  and  smoothing filter (right)

Figure 4. 3D plot of ‘texture 1’ after Gabor filter

Figure 5. 3D plot of ‘texture 1’ after Gabor filter and smoothing filter

Figure 6. superimposition of texture segmentation of ‘texture1’

image2:

Figure 7. binary image of ‘texture 2’

Figure 8. grayscale image of Gabor filter (left)  and smoothing filter (right)

Figure 4. 3D plot of ‘texture 1’ after Gabor filter

Figure 5. 3D plot of ‘texture 1’ after Gabor filter and smoothing filter

Figure 6. superimposition of texture segmentation of ‘d9d77’

Observation:

image3 :

Figure 3. grayscale image of Gabor filter (left)  and smoothing filter (right)

Figure 4. 3D plot of ‘texture 1’ after Gabor filter

Observation:

image4:

Figure 3. grayscale image of Gabor filter (left)  and smoothing filter (right)

Figure 4. 3D plot of ‘texture 1’ after Gabor filter

Figure 5. 3D plot of ‘texture 1’ after Gabor filter and smoothing filter

Observation:

- Conclusion

In conclusion, the project